Faculty of Engineering

Systems & Computers Eng., Dept.,

Subject: Computer Architecture

Sheet # 2

- A 64K × 16 RAM chip uses coincident decoding by splitting the internal decoder into row select and column select. (a) Assuming that the RAM cell array is square, what is the size of each decoder, and how many AND gates are required for decoding an address? (b) Determine the row and column selection lines that are enabled when the input address is the binary equivalent of (32000)10.

- A DRAM has 15 address pins and its row address is 1 bit longer than its column address. How many addresses, total, does the DRAM have?

- A 1 Gb DRAM uses 4-bit data and has equal-length row and column addresses. How many address pins does the DRAM have?

- A DRAM has a refresh interval of 128 ms and has 4096 rows. What is the interval between refreshes for distributed refresh? What is the total time required out of the 128 ms for a refresh of the entire DRAM? What is the minimum number of address pins on the DRAM?

- Using the 64K × 8 RAM chip in Figure 9 plus a decoder, construct the block diagram for a 512K × 16 RAM. 10

- Explain how SDRAM takes advantage of the two-dimensional storage array to provide a high data access rate?